SMART MASJID

FIRMWARE REQUIREMENT SPECIFICATION (FRS)

- There will be two types of devices.

- Button

- Speaker

- Upon first boot up since the WIFI credentials are not available, the device shall enable Bluetooth and go to a configuration state to acquire the following:

  - Credentials required for WIFI Connection.

  - An ID named Masjid ID. (For Button Only)

And store them in ROM.

- The device shall acquire an IP address from a WIFI router using the DHCP protocol.

- Upon turning on, if the device acquires WIFI credentials from the stored data, it shall connect to the WIFI router using DHCP protocol, keeping the Bluetooth disabled.

- In case of failure occurs in making a connection with the WIFI Router due to any unknown reasons then the device shall simply enable the Bluetooth and go to a configuration state.

- Upon getting the IP address the device shall transmit the telemetry of the device which includes:

  - Battery health

  - MAC address

- The device shall keep trying to send the telemetry a given number of times if the request is unsuccessful.

- The device shall then connect to the MQTT broker and subscribe to the following topics.

  - Subscribe (For Speaker Only)

  - Unsubscribe (For Speaker Only)

  - WIFI

  - Reset

- The device shall then subscribe to the topics stored in ROM. (For Button Only)

- The device shall send an HTTP POST request to initiate the adhan streaming when the button is pressed. (For Button Only)

- The device shall send an HTTP POST request to stop the adhan if the button is pressed and the adhan is streaming. (For Button Only)

- If the button is not pressed within 5 minutes of the adhan initiation, the device shall stop the adhan by sending an HTTP POST request. (For Button Only)

- The device shall receive the topics to subscribe and unsubscribe to in their respective MQTT topics. The topics will be adhans. (For Speaker Only)

- The device shall store the topics in ROM which are received in the subscribe topic of MQTT. (For Speaker Only)

- The device shall delete the topics in ROM which are received in the unsubscribe topic of MQTT. (For Speaker Only)

- The device shall receive the adhan streaming in the respective adhan topic and play that stream using the speaker. (For Speaker Only)

- The device shall remove the WIFI credentials from ROM if it receives something in the WIFI topic of MQTT.

- The device then tries to connect to the credentials received in the payload.

- In case of a given number of failed attempts the device should enable Bluetooth and go to configuration mode.

- The device shall completely reset itself by erasing all stored data from ROM when received something in RESET topic of MQTT.